HALFEN USA Inc.		March 2012

8521 FM 1976

P.O. Box 547

Converse, TX 78109

Toll Free	800-423-9140

Fax	888-277-1695

Website	www.halfenusa.com

E-mail	info@halfenusa.com

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, including MasterFormat, SectionFormat, and PageFormat, as described in The CSI Construction Specifications Practice Guide.

This section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Section numbers are from MasterFormat 2011 Update.

- 08 44 00

CURTAIN WALL ASSEMBLIES FOR TOP OR EDGE OF SLABS

Specifier Notes:  This section covers Halfen site-installed, curtain wall assemblies to secure vertical glazed walls (such as curtain wall framing members) in position.  A three-dimensional adjustment can be achieved with the anchor channels.  Consult Halfen USA for assistance in editing this section for the specific application.

  - GENERAL

    - SECTION INCLUDES

Specifier Notes:  Edit the following for the specific application.

      - Curtain wall assemblies attach to substrate for secure attachment.

    - RELATED REQUIREMENTS

Specifier Notes:  Edit the following list of related sections as necessary.  Limit the list to sections with specific information that the reader might expect to find in this section, but is specified elsewhere.

      - Section 03 15 00 – Concrete Accessories:  Anchor channel providing structural support for curtain wall assemblies.

      - Section 05 05 23 – Metal Fastenings:  Fasteners to secure curtain wall anchors.

    - REFERENCE STANDARDS

Specifier Notes:  List standards referenced in this section, complete with designations and titles.  Delete standards not included in the edited section.  Including a standard in this list does not require compliance with that standard.

      - ASTM A1011/A1011M – Standard Specification for Steel, Sheet and Strip, Hot-Rolled, Carbon, Structural, High-Strength Low-Alloy, High-Strength Low-Alloy with Improved Formability, and Ultra-High Strength.

      - ASTM B221 – Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes.

    - COORDINATION

      - Coordinate the Work with installation of curtain wall mullion assembly onto structural supporting components.

    - PREINSTALLATION MEETINGS

Specifier Notes:  Edit preinstallation meetings as necessary.  Delete if not required.

      - Convene preinstallation meeting [1 week]  [2 weeks] before start of work of this section.

      - Require attendance of parties directly affecting work of this section, including Contractor, Architect, Engineer, installer, and manufacturer’s representative.

      - Review materials, preparation, installation, tolerances, protection, and coordination with other work.

    - SUBMITTALS

Specifier Notes:  Edit submittal requirements as necessary.  Delete submittals not required.

      - Comply with Section 01 33 00 – Submittal Procedures.

      - Product Data:  Submit manufacturer’s product data.

      - Shop Drawings:  Submit manufacturer’s shop drawings, indicating component profiles, sizes, connection attachments, anchorage, size and type of fasteners, and accessories.

      - Manufacturer’s Certification:  Submit manufacturer’s certification that materials comply with specified requirements and are suitable for intended application.

      - Test Reports:  Submit manufacturer’s substantiating engineering data, test results of previous test by independent laboratory which meet performance criteria, and other supportive data.

      - Warranty Documentation:  Submit manufacturer’s standard warranty.

    - QUALITY ASSURANCE

      - Manufacturer’s Qualifications:

        - Capable of providing field service representation during installation.

        - Minimum of 5 years experience in manufacture of curtain wall assemblies.

        - Experience in projects of similar scope.

        - Manufacture in accordance with established quality assurance program.

    - DELIVERY, STORAGE, AND HANDLING

      - Delivery and Acceptance Requirements:  Deliver materials to site in manufacturer’s original, unopened containers and packaging, with labels clearly identifying product name and manufacturer.

      - Storage and Handling Requirements:

        - Store and handle materials in accordance with manufacturer’s instructions.

        - Keep materials in manufacturer’s original, unopened containers and packaging until installation.

        - Store materials in clean, dry area indoors.

        - Protect materials and finish during storage, handling, and installation to prevent damage.

  - PRODUCTS

    - MANUFACTURER

      - Halfen USA Inc P.O. Box 18687, San Antonio, TX 78218.  Toll Free 800-423-9140.  Fax 888-277-1695.  www.halfenusa.com.  info@halfenusa.com.

    - DESIGN CRITERIA

Specifier Notes:  Restrict statements to describe the combined result of the components used to assemble the system.  If more than one type and weight of load is anticipated, create a schedule of loads, types of anchors, and locations.

      - Curtain Wall Assembly:  Anchor plates and attachment shall resist loads as follows, each without damage or permanent set:

        - Dead (Shear) Load:  _______ lbs (_______ kN).

        - Wind/Suction Load:  _______ lbs (_______ kN).

    - MATERIALS

Specifier Notes:  Edit the following paragraph to suit placement in concrete formwork for casting in place using stud-type curtain wall anchors.

      - Edge of Slab Brackets:  Halfen multi-adjustable curtain wall clips.

        - Bracket Style:  Halfen multi-adjustable curtain wall clips.

        - Bracket Material:  ASTM B221, Alloy 6005, Temper T5, extruded aluminum, with serrated faces.

Specifier Notes:  Select one or both of the following attachment devices as required to suit project conditions.

        - Anchors Within Concrete Substrate:  Embedded slotted channel with:

          - T-head adjusting bolts.

          - Serrated washers to match channel serrations.

          - Locking nut.

        - Anchors Penetrating Substrate:

          - Drill holes for anchors.

          - Set and impress anchors securely in place.

          - Hex-head bolts.

          - Lock washers.

          - Locking nut.

      - Top of Slabs:

        - Bracket Style: Halfen top of slab assembly.

        - Bracket Material:  Steel, ASTM A1011.

        - Anchors Within Concrete Substrate:  Embedded slotted channel with:

          - T-head adjusting bolts.

          - Serrated washers to match channel serrations.

          - Locking nut.

        - Anchors Penetrating Substrate:

          - Drill holes for anchors.

          - Set and impress anchors securely in place.

          - Hex-head bolts.

          - Lock washers.

          - Locking nut.

    - FABRICATION

      - Shop assemble and package items for delivery to site, in variety of sizes required.

  - EXECUTION

    - EXAMINATION

      - Examine areas and supports to receive curtain wall anchors.

      - Verify dimensions, tolerances, and method of attachment with other work.

      - Notify Architect of conditions that would adversely affect installation.

      - Do not begin installation until unacceptable conditions are corrected.

    - PREPARATION

      - Supply items required to be anchored to substrate with setting templates to appropriate sections.

    - INSTALLATION

      - Install curtain wall anchors in accordance with manufacturer’s instructions at locations indicated on the Drawings.

      - Install items plumb and level, accurately fitted, free from distortion or defects.

      - Obtain approval from Architect and manufacturer before site cutting or making adjustments not scheduled.

    - TOLERANCES

Specifier Notes:  Edit the following sentence.

      - Edge of Slab Assembly Maximum Variation from [Plumb]  [Level]:  [1/8]  [1/4]  [______] inch ([3]  [6]  [______] mm).

      - Top of Slab Assembly Maximum Variation from [Plumb]  [Level]:  [1/8]  [1/4]  [______] inch ([3]  [6]  [______] mm).

    - PROTECTION

      - Protect installed curtain wall anchors from damage during construction.

END OF SECTION